T3 - TEARCI

Tomás Carvalho, N16

Questions and Answers

GROUP 4 - Audio Power Amplifier

1- Describe each functional block of the amplifier

The amplifier is divided into three main functional blocks. The differential input stage receives the audio signal and the negative feedback signal, compares them, and generates an error signal while providing the initial voltage gain. Next, the voltage amplifier stage (VAS) is responsible for most of the amplifier’s total voltage gain and for producing the voltage swing required to drive the following stage. Finally, the output stage operates as a current buffer, supplying high current gain to drive a low-impedance load such as a loudspeaker.

In addition to these stages, there are supporting blocks such as the bias network,ets the quiescent current to reduce crossover distortion, and the frequency compensation network, which ensures the overall stability of the amplifier.

2- Explain the role of the differential input stage

The differential input stage serves as the front end of the amplifier. Its primary function is to compare the input signal with the negative feedback signal. By amplifying the difference between them (the error signal), it corrects deviations from the intended output. This stage also provides the initial voltage gain and presents a high input impedance, reducing the load on the signal source. The circuit is designed in an inverting configuration, which helps simplify stability requirements when compared to non-inverting configurations.

3- Describe where is the amplifying voltage stage and current amplifying stage

The Voltage Amplifying Stage (VAS) comes directly after the differential input stage. It is responsible for producing a large voltage swing and supplies most of the amplifier’s overall voltage gain. The Current Amplifying Stage is the final output stage and is configured as a push-pull emitter follower. Its role is to deliver the high current gain required to drive the loudspeaker, while maintaining a voltage gain close to unity.

4- Explain how the VAS and miller capacitor ensure stability

The Miller capacitor is placed across the Voltage Amplifier Stage (VAS). Because the VAS has a high gain, the capacitor’s effective value is multiplied by the Miller effect, resulting in a much larger equivalent capacitance. This creates a dominant pole in the amplifier’s frequency response. The resulting controlled roll-off ensures that the open-loop gain falls to unity before the phase shift reaches 180 degrees, thus maintaining an adequate phase margin (for example, around 40 degrees) and preventing high-frequency oscillations.

5- Describe the current and voltage behavior of the darlington output stage

The Darlington output stage works as an emitter follower, so its voltage gain is approximately unity (Av ≈ 1). The output voltage closely follows the voltage provided by the VAS. Its main purpose is current amplification, acting as a buffer. By using the high current gain of the power transistors, a low-current signal from the VAS is able to control the large current drawn from the power supply rails and delivered to the loudspeaker, operating in a push-pull configuration.

6- What is the working class of this amplifier?

The amplifier operates in Class AB mode. This can be inferred from the use of a small quiescent current established by the bias network. This low but non-zero current keeps both output transistors slightly conducting even when no input signal is present, effectively eliminating crossover distortion, which is typical in Class B operation, while still achieving higher efficiency than a pure Class A amplifier.

7- Detail power, thermal, and layout consideration

Power: The 60 W output power demands a robust dual-rail power supply capable of delivering high current.
Thermal: Adequate heatsinking is required for the TO-220 output power transistors when operating at the full 60 W level, in order to prevent overheating and thermal runaway. The low quiescent current helps to limit static power dissipation.
Layout: A clean and well-organized PCB layout is essential to reduce unwanted signal coupling, parasitic capacitances, and inductances. This is particularly important for maintaining stability and performance at high frequencies. The use of low-inductance parallel emitter resistors is a layout-related design choice that further improves stability.