T3 - TEARCI

Tomás Carvalho, N16

GROUP 3 - DHT22–LCD

C-1 – Temperature Sensor (DHT22): Function and Operating Principle
The DHT22 sensor is responsible for measuring the ambient temperature and relative humidity of the system.
It integrates calibrated sensing elements and transmits the measured data to the microcontroller as a digital signal through a single-wire communication interface.

C-2 – LCD with I2C Interface
The LCD module provides the user interface by displaying temperature values, system status, and operational messages.
The use of the I²C communication protocol reduces wiring complexity, minimizes the number of required microcontroller I/O pins, and allows easy system expansion.

C-3 – Hysteresis Control Logic
Hysteresis defines the difference between the activation and deactivation temperature thresholds of the control system.
It prevents rapid output switching when the measured temperature is close to the setpoint, thereby improving control stability and increasing component lifetime.

C-4 – State Memory Variable
A state memory variable is required to store the current output condition (ON/OFF).
This ensures deterministic system behavior and avoids oscillation, especially when hysteresis control is implemented.

C-5 – Output Stage (Digital Pin D3)
A relay cannot be driven directly from a microcontroller output pin due to current limitations and inductive voltage transients.
An interface circuit is therefore required, typically consisting of a transistor driver, a flyback protection diode, and optionally an opto-isolator for electrical isolation.

C-6 – Reliability and Safety
Sensor malfunction or invalid readings can result in unsafe operating conditions such as overheating or failure to activate the output.
Safety improvements include validation of sensor data, implementation of safe default states, alarm signaling, and enforcement of maximum and minimum temperature limits.