T3 - TEARCI

Tomás Carvalho, N16

GROUP 3 - EV_ParkingSensor_Shield

C-1 – Ultrasonic Measurement Principle

The HC-SR04 measures distance using time of flight. It emits an ultrasonic pulse and measures the time until the echo returns from an object.
Distance = (time × speed of sound) / 2.
The conversion cm = µs / 58 comes from the speed of sound in air (~343 m/s) and accounts for the round trip.

C-2 – Why Use a Timeout in pulse?

A timeout prevents the system from blocking indefinitely if no echo is received.
Without a timeout, the program can freeze, causing missed updates or unsafe behavior.
This matters in real systems where sensors can fail or echoes may not occur.

C-3 – LED Bar-Gauge Mapping

Mapping distance into multiple levels gives progressive, intuitive feedback instead of a binary on/off signal.
It improves usability, allows quicker interpretation, and supports better human-machine interaction by showing proximity trends.

C-4 – Colour Zones and Thresholds

Green/yellow/red follow human risk perception conventions: safe - caution - danger.

Example thresholds for a small EV:

Green: >30 cm

Yellow: 10–30 cm

Red: <10 cm

This reduces reaction time and cognitive load.

C-5 – Buzzer Interface and Safety

A passive piezo is tone-appropriate because frequency can be controlled in software.
An active buzzer generates its own tone and only needs on/off control, but offers less flexibility.
Active buzzers simplify hardware but reduce feedback customization.

C-6 – Reliability and Safety

Failure modes:

Wiring faults (loose or broken connections);

Supply noise or voltage drops;

Poor reflections from angled or soft surfaces.

Improvements:

Signal filtering or averaging;

Proper mechanical alignment;

Shielded cables and stable mounting.

C-7 – Automotive Context

Sensors should be mounted on the front or rear bumper, facing forward/backward.
Constraints include:

Correct angle to avoid ground reflections

Resistance to vibration

Protection against water, sunlight, and temperature extremes