T3 - TEARCI

Tomás Carvalho, N16

RobortArm Joystick - Group 3

# – Joystick Analogue Input

Joystick axes require analogue inputs because they output a continuous range of voltage values rather than just ON or OFF states. The Arduino’s analogue inputs read these voltages as values from 0 to 1023. When the joystick is in the centre position, the reading is approximately 512, since it is powered by 5 V and the middle position corresponds to about half of that voltage.

# – Deadzone Concept

A deadzone is a small neutral region around the joystick’s centre position where minor movements are ignored. It is necessary because joysticks are not perfectly stable and may produce slight voltage variations even when they are not being touched. The deadzone helps prevent unintended movements and ensures smoother, more stable control.

# – Short Press vs Long Press

The sketch determines how long the button is held by using time functions such as millis. A short press is identified when the button is released before a set time limit, while a long press is detected when the press duration goes beyond a defined threshold. Debouncing is necessary to prevent false activations caused by the mechanical bouncing of the button’s contacts.

# – Servo Control Limits

Servos must be restricted to a range between 0° and 180° because this is their physical operating limit. If these boundaries are not respected, the servo may attempt to rotate beyond its mechanical constraints, leading to jitter, overheating, gear damage, or even permanent failure.

# – LCD Parallel Interface

Using 4-bit mode instead of 8-bit mode reduces the number of Arduino pins needed to control the LCD. This leaves more pins available for other components, such as servos and the joystick, while still maintaining reliable and adequate display performance.

# – Power Integrity

Servos can cause the Arduino to reset because they draw a high current, especially at startup or under load, which can lead to a drop in the supply voltage. Two practical solutions are powering the servos with an external power supply and adding decoupling capacitors to the power lines.