Mini-Project Notes: Short-Range Infrared Light Barrier

Student: Tomás Carvalho | Course: T3-TEARCI | No.: 16

Notes on the Objectives of the Activity

1) Objective of the Activity and Brief Explanation of the Components Used

The objective of this activity was to build an infrared transmitter circuit.

# 555 Timer

The circuit uses the 555 Timer in Astable Mode, where it acts as a continuous square-wave generator that switches ON and OFF repeatedly. This defines the signal frequency, which is why the 555 is often called the “rhythm generator.”

# Additional Components Used

# Resistors;

Capacitor;

These components determine the maximum and minimum frequencies (F_max and F_min) and the Duty Cycle (D) of the output pulse.

Infrared LEDs and Current Behavior

The circuit was designed with a 13% duty cycle for efficiency.

## Peak Current (I_peak)

This is the maximum current that flows when the LED is ON (13% of the time).  A high peak current increases the infrared transmission range.

## Average Current (I_avg)

This is the total current consumed over the entire cycle. It is low because:

I_avg = I_peak × 0.13

A low average current saves battery energy and ensures high efficiency.

## Conclusion

A low duty cycle allows the LED to produce a strong infrared flash while keeping the overall power consumption very low.

Function of Additional Components

## 1. BD140 Transistor (Amplification and Switching)

The BD140 is essential because the 555 Timer alone cannot supply the high current required for long-range infrared transmission.

Functions:

Switching:

The transistor works as an electronic switch. The small signal from the 555 turns the transistor ON and OFF, controlling the high current flowing to the LEDs.

Amplification:

It amplifies the small base current coming from the 555.

Gain:

This is the multiplication factor of the transistor. A higher gain allows a small base current to control a much larger collector current.

# Base Current and Base Terminal

Base Current (I_base):

The small current that enters the base terminal and controls the much larger collector-toemitter current.

Base Terminal:

The transistor terminal responsible for regulating the current flow from collector to emitter.

## 2. Capacitor C2 (Power Supply Stabilization)

The large electrolytic capacitor acts like an energy reservoir.

Its function:

It stabilizes the power supply voltage.

When the LEDs turn ON with high I_peak, they draw current quickly, which could cause a voltage drop.

C2 delivers the extra energy during the pulses, keeping the voltage stable and preventing interference in the 555 Timer.

# General Summary

Overall, the circuit works as a team:

The 555 generates the rhythm, the transistor amplifies and switches the current, and C2 ensures the power supply remains stable.

Calculations and Formulas

- – Output Frequency (f)

Number of pulses per second, measured in Hertz (Hz).

- – Duty Cycle

Percentage of time the 555 output stays HIGH during a full cycle.

- – Average Current (I_avg)

The actual current consumed over time, and the main factor determining battery usage.

# 4 – Current Gain Factor

The current multiplication factor that shows how the small 555 base signal controls the high LED current.

I_collector: High current delivered to the LEDs (close to I_peak).

I_base: Small signal current provided by the 555 Timer.

Example:

If the transistor has a beta of 100, it can switch 100 mA of collector current using only 1 mA of base current.

# FORMULAS AND CALCULATIONS